| Constant Heading Ship: |  |
| --- | --- |
| Maneuvering Ship: |  |
| Name of Designated POAC: |  |
| Name of STS Superintendent if Different from POAC |  |
| Date of Transfer |  |
| Location of Transfer |  |

For discharging / receiving ship answer as appropriate)

| CL 4A | Generic Checks | CHS Status | MS Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Local requirements including permissions are obtained and complied with OCIMF | ☐Yes | ☐Yes | ☐ Not applicable |
| 2 | JPO procedures for cargo and ballast operations reviewed and agreed by all parties. | ☐Yes | ☐ Yes |  |
| 3 | Present and forecast weather and sea conditions are within the agreed limits. | ☐Yes | ☐Yes |  |
| 4 | Cargo specifications, hazardous properties, SDS and any requirements for inerting, heating, reactivity and inhibitors have been exchanged. | ☐Yes | ☐Yes | ☐ Not applicable |
| 5 | Tank venting system and closed operation procedures agreed | ☐Yes | ☐Yes |  |
| 6 | Procedures for vapour control/balancing have been agreed. | ☐Yes | ☐Yes | ☐ Not applicable |
| 7 | Procedures for the transfer of personnel have been agreed. | ☐Yes | ☐Yes | ☐ Not applicable |
| 8 | All personnel engaged in the cargo operation are provided with PPE including, where necessary, personal gas detectors/monitors in accordance with the ship operator's PPE matrix. | ☐Yes | ☐Yes |  |
| 9 | Cargo transfer and, if applicable, the vapour return equipment is: In good condition Of the appropriate type Properly fitted with gaskets/seals Lined up correctly Properly rigged Secured to the manifolds Sufficiently supported | ☐Yes | ☐Yes |  |
| 10 | Electrical insulation of the ship/ship interface is effective. | ☐Yes | ☐Yes |  |
| 11 | Effective STS communications established. | ☐Yes | ☐Yes |  |
| 12 | Emergency signals and shutdown procedures are agreed. | ☐Yes | ☐Yes |  |
| 13 | The engine room will be manned and the main engine kept on standby or on short notice of readiness | ☐Yes | ☐Yes |  |
| 14 | Officers in charge of the cargo transfer on both ships are identified, and details have been exchanged and posted. | ☐Yes | ☐Yes |  |
| 15 | Procedures for cargo and ballast handling operations and transfer parameters agreed. | ☐Yes | ☐Yes |  |
| 16 | Simultaneous Operations (SIMOPS) are identified and agreed | ☐Yes | ☐Yes |  |
|  | - Nitrogen purging or inerting | ☐Yes | ☐Yes |  |
|  | - Repairs/maintenance | ☐Yes | ☐Yes |  |
|  | - Tank cleaning | ☐Yes | ☐Yes |  |
|  | - COW | ☐Yes | ☐Yes |  |
|  | - Slops discharge | ☐Yes | ☐Yes |  |
|  | - Waste discharge | ☐Yes | ☐Yes |  |
|  | - Bunkering | ☐Yes | ☐Yes |  |
|  | - Receiving stores | ☐Yes | ☐Yes |  |
|  | - Personnel transfer | ☐Yes | ☐Yes |  |
|  | - Crew change | ☐Yes | ☐Yes |  |
|  | - Planned drills | ☐Yes | ☐Yes |  |
| 17 | Messengers and toggle pins are prepared and positioned ready for unmooring in accordance with the unmooring plan. | ☐Yes | ☐Yes |  |
| 18 | Means of emergency escape from both ships are established. | ☐Yes | ☐Yes |  |
| 19 | STS operation supervision and watchkeeping is adequate | ☐Yes | ☐Yes |  |
| 20 | There are sufficient personnel to deal with an emergency . | ☐Yes | ☐Yes |  |
| 21 | Naked lights, smoking restrictions and designated smoking areas are established | ☐Yes | ☐Yes |  |
| 22 | Control of electrical and electronic devices is agreed | ☐Yes | ☐Yes |  |
| 23 | Routine for regular checks and exchange of information on cargo transferred are agreed. | ☐Yes | ☐Yes | CL 5B, 5C: repetitive checks |
| 24 | The procedure for stopping transfer are agreed | ☐Yes | ☐Yes |  |
| 25 | Cargo and vapour balancing hoses are supported and protected from chafing and the hose release area is clear of obstructions. | ☐Yes | ☐Yes |  |
| 26 | Tools required for rapid disconnection are located at the cargo manifold. | ☐Yes | ☐Yes |  |

| CL 4B | Additional for Vapor Balancing | CHS Status | MS Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Confirm ship's vapour headers, manifolds and all piping are drained of liquid | ☐Yes | ☐Yes |  |
| 2 | Confirm all vapour headers are vapour tight before hose connection | ☐Yes | ☐Yes |  |
| 3 | State pressure in cargo tanks (mm WG or psi) | ☐Yes | ☐Yes |  |
| 4 | Tank oxygen content is below 8% by volume | ☐Yes | ☐Yes |  |
| 5 | Tank high-level and overfill alarms have been tested within the last 24 hours | ☐Yes | ☐Yes |  |
| 6 | Vapour recovery/balancing valves are correctly set | ☐Yes | ☐Yes |  |
| 7 | Vapour recovery hose is inerted, where needed | ☐Yes | ☐Yes |  |
| 8 | Vapour hose connection confirmed vapour tight | ☐Yes | ☐Yes |  |
| 9 | Ship pressure and vacuum relief settings in: Liquid P/V breaker Tank P/V valves Mast head P/V valve(s) Other P/V relieving devices | ☐Yes | ☐Yes |  |
| 10 | Type of vapours from current/previous cargoes | ☐Yes | ☐Yes |  |
| 11 | Are the vapours being balanced harmful to ship's crew? E.g. H2​S | ☐Yes | ☐Yes |  |
| 12 | Maximum pressure differential at maximum transfer rate (mm WG or psi) | ☐Yes | ☐Yes |  |
| 13 | Cargo tank pressure range to be maintained (mm WG or psi) | ☐Yes | ☐Yes |  |
| 14 | Cargo tank pressure alarm set points High alarm (mm WG or psi Low alarm (mm WG or psi) | ☐Yes | ☐Yes |  |
| 15 | IG main pressure alarm set points: High alarm (mm WG or psi) Low alarm (mm WG or psi) | ☐Yes | ☐Yes |  |
| 16 | Vapour emission control system pressure alarm set points: High alarm (mm WG or psi) Low alarm (mm WG or psi). | ☐Yes | ☐Yes |  |
| 17 | Oxygen analyser has been checked and calibrated | ☐Yes | ☐Yes |  |
| 18 | Agreement on the transfer sequence and procedures: Normal start up. Normal shutdown. Low vapour pressure alarm. High vapour pressure alarm. | ☐Yes | ☐Yes |  |

| CL 4C | Additional for Chemical tankers | CHS Status | MS Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Inhibition certificate received (if required) from manufacturer | ☐Yes | ☐Yes | Not applicable |
| 2 | Countermeasures against personal contact with cargo are agreed | ☐Yes | ☐Yes |  |

| CL 4D | Additional for LPG and LNG transfer | CHS Status | MS Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Inhibition certificate received (if required) from manufacturer | ☐Yes | ☐Yes | Not applicable |
| 2 | Maximum working pressures are agreed between ships | ☐Yes | ☐Yes |  |
| 3 | Cargo handling rate and relationship with valve closure times and automatic shutdown systems is agreed | ☐Yes | ☐Yes |  |
| 4 | Maximum/minimum temperatures/pressures of the cargo to be transferred are agreed | ☐Yes | ☐Yes |  |
| 5 | Cargo tank relief valve settings are confirmed | ☐Yes | ☐Yes | Record relief valve settings: CHS: MS: |
| 6 | Cooldown procedures have been agreed | ☐Yes | ☐Yes |  |
| 7 | Procedures for increasing/reducing transfer rates have been agreed | ☐Yes | ☐Yes |  |
| 8 | The potential for cargo roll-over has been considered | ☐Yes | ☐Yes |  |
| 9 | The deck watch is aware of the location and activation method of ESD systems on deck | ☐Yes | ☐Yes |  |

| CL 4E | Additional for LNG transfer | CHS Status | MS Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | ESD and ERS system arrangements are in place and tested: ESD warm test has been undertaken from both ships. ERS release mechanism functional test only (with no coupling release) has been tested. | ☐Yes | ☐Yes |  |
| 2 | Cargo transfer lines have been purged with nitrogen to below 5%O2​ | ☐Yes | ☐Yes |  |
| 3 | Cargo transfer line connections are leak tested | ☐Yes | ☐Yes |  |
| 4 | The nitrogen plant will be operational throughout the transfer | ☐Yes | ☐Yes |  |
| 5 | The protective water curtain is running | ☐Yes | ☐Yes |  |

| CL 4F Pretransfer Agreements |  |  |  |
| --- | --- | --- | --- |
| S. No |  | Description | Agreement |
| 1 | Latest version of the JPO |  | Date/version JPO: |
| 2 | Working language |  |  |
| 3 | Agreed SIMOPS |  |  |
| 4 | Ships ready for maneuvering |  | ☐ Not applicable for heading ships |
|  |  |  | Notice period (maximum for full readiness to maneuver): |
|  |  |  | Ship 1: _ ____ min. |
|  |  |  | Ship 2: __ ___ min. |
| 5 | Agreed communication system and backup arrangement |  | Primary system: |
|  |  |  | Backup system: |
| 6 | Operational supervision and watchkeeping |  | Ship 1 responsible persons: |
|  |  |  | Ship 2 responsible persons: |
|  |  |  | Terminal (if applicable) responsible persons: |
| 7 | Dedicated smoking areas and naked light restrictions |  | Ship 1 restrictions: |
|  |  |  | Ship 2 restrictions: |
|  |  |  | Terminal (if applicable) restrictions: |

| 8 | Maximum wind, current and sea/swell criteria or other limiting environmental factors | Stop cargo transfer: |
| --- | --- | --- |
|  |  | Maximum wind speed: |
|  |  | Current: |
|  |  | Swell: |
|  |  | Disconnect: |
|  |  | Unmooring: |
| 9 | Limits for cargo and ballast handling | Maximum transfer rates: |
|  |  | Topping-off rates: |
|  |  | Maximum manifold pressure: |
|  |  | Cargo temperature: |
|  |  | Other limitations: |
| 10 | Pressure surge control | Loading ship: |
|  |  | Minimum number of cargo tanks open: |
|  |  | Tank switching protocols: |
|  |  | Full load rate: |
|  |  | Topping off-rate: |
|  |  | Closing time automatic valves: |
| 11 | Cargo transfer management | Action notice periods: |
|  |  | Transfer stop protocols: |
| 12 | Routine for regular checks on cargo transferred are agreed | Routine transferred quantity checks: |
| 13 | Emergency signals | Ship 1 signal: |
|  |  | Ship 2 signal: |
|  |  | Terminal (if applicable) signal: |
| 14 | Tank system | Ship 1 system: |
|  |  | Ship 2 system: |
| 15 | Closed operations | ☐ Not applicable |
|  |  | Requirements: |
| 16 | ESD (oil and chemical) | ☐ Not applicable |
|  |  | Confirm ESD system |
| 17 | ESD and ERS systems (LPG and LNG) | ☐ Not applicable |
|  |  | Fibre optic/electrical link |
|  |  | Closing time ESD valve unloading ship: seconds |
|  |  | Closing time ESD valve loading ship: seconds |
|  |  | ERS ☐ Yes ☐ No |
| 18 | In case of vapour balancing | ☐ Not applicable |
|  |  | Which vessel will vent, if needed? |

| Name: |  |
| --- | --- |
| Rank: |  |
| Signature: | Date: |