| Constant Heading Ship: |  |
| --- | --- |
| Maneuvering Ship: |  |
| Name of Designated POAC: |  |
| Name of STS Superintendent if Different from POAC: |  |
| Date of Transfer: |  |
| Location of Transfer: |  |

CL :6A– CHECKS AFTER TRANSFER BEFORE DISCONNECTION

| CL 6A | Checks before Disconnection | Status | Remarks |
| --- | --- | --- | --- |
| 1 | Cargo hoses, fixed cargo pipelines, vapour return lines and manifolds are drained and confirmed to be liquid-free | ☐ Yes |  |
| 2 | Cargo hoses, vapour return lines, fixed pipelines and manifolds are: Purged. Inverted. Depressurized. | ☐ Yes |  |
| 3 | All remotely and manually operated valves are closed ready for disconnection | ☐ Yes |  |
| 4 | Sufficient personnel with responsible officer available for disconnection | ☐ Yes |  |
| 5 | Correct PPE is used | ☐ Yes |  |
| 6 | The other ship is notified on "ready to disconnect" | ☐ Yes |  |

CL :6B– CHECKS AFTER DISCONNECTION BEFORE UNMOORING

| CL 6B | Checks before Unmooring | Status | Remarks |
| --- | --- | --- | --- |
| 1 | Cargo hoses and/or manifolds are securely blanked | ☐ Yes |  |
| 2 | Cargo area on the ship is cleared and restored to standard condition | ☐ Yes |  |
| 3 | Cargo documents signed and exchanged | ☐ Yes |  |
| 4 | Terminal or transfer location authority is notified on the completion of the STS operation | ☐ Yes | ☐ Not applicable |
| 5 | The transfer side of the ship is clear of obstructions (including hose lifting equipment) | ☐ Yes |  |
| 6 | The method of letting go of moorings and separation of ships has been agreed | ☐ Yes |  |
| 7 | Mooring winches ready for operation | ☐ Yes |  |
| 8 | Rope messengers and stoppers are available at mooring stations | ☐ Yes |  |
| 9 | Communications are established with mooring personnel and with the other ship | ☐ Yes |  |
| 10 | Shipping traffic in the area is being monitored and a VHF alert has been transmitted | ☐ Yes |  |
| 11 | Manoeuvring, mooring and navigational equipment has been tested and is ready for departure | ☐ Yes |  |
| 12 | The other ship has been notified that unmooring can commence | ☐ Yes |  |

| Officer in charge of CHS: | Name: |
| --- | --- |
| Officer in charge of MS: | Name: |
| Terminal: | Name: |
| STS Supdt: | Name: |