Joint Plan of Operation (Fujairah)

## Part A - Operation & Location Specific Information

This JPO is prepared by Oceane Group of companies and  both the vessels should comply with the JPO proposed herewith.

| Maneuvering Vessel: | Enter Name of Vessel | Vessel at Anchor: | Enter Vessel name |
| --- | --- | --- | --- |
| POAC: | Enter POAC Name | Proposed Date of Transfer: | Choose Date |

These instructions are applicable to all vessels during 	STS operation. In all phases of the operation the Masters of the involved vessels shall remain solely responsible for the safety and proper navigation of their vessels and for all aspects of the cargo transfer operations conducted by their vessels. The Mooring Master is to act solely in an advisory capacity to assist the Masters of the vessels, as necessary, in the mooring, cargo transfer and unmooring operations related to the STS transfer. The presence of the Mooring Master shall not relieve the Masters of the vessels of their responsibilities. Oceane Marine or the Mooring Master nor any other person engaged by Oceane Fenders, in connection with the performance of STS Transfer Services, shall be liable to the Master, Owners, Operators, Ship Managers and Charterers (if any) or to any other party for any loss, damage, or delay to the vessel and/or her cargo, even if such loss damage or delay is caused by the negligence of Oceane Fenders, Mooring Master or any such other person.

Specific instructions and requirements relating to the STS cargo and operation are to be supplied in the

- Kindly proceed to Fujairah ‘S’ Anchorage, and anchor after taking permission from Port Control on VHF Ch. 10/ 16. Keeping safety of operation as paramount, the attending POAC may shift the above-mentioned rendezvous to suit local conditions prevailing at the time and vessel will be communicated in this regard.  The attending POAC will contact your vessel prior to arrival, and advice regarding the ETA and name the service craft. The primary (one string consist of two fenders and one fender) and secondary fenders will be rigged on Enter Vessel name on her Stbd. side at “S” anchorage. STS gear will be brought to the rendezvous by workboat. Vessels will be contacted by workboat via VHF to arrange exact position, and any requirements regarding lee, heading and speed. After completing operations with the workboat, the Maneuvering vessel will proceed to a suitable initial position as agreed between Port Pilot and POAC (Person Overall Advisory Control) and both the master’s to commence operation. Port Pilot will board Enter Name of Vessel and Berthing will normally be conducted by Port Pilot and Tugs while Enter Vessel name at anchor the heading and speed will be dependent on weather conditions and traffic density at the time. Once all fast, cargo transfer hoses will be connected, and pre-transfer tests are conducted. Cargo transfer will be conducted at anchor in “S” anchorage. On completion of cargo transfer, hoses will be drained and then returned to maneuvering vessel or left on non-maneuvering vessel ready for subsequent operations.

- On completion of post cargo transfer operations, both the vessels will be separated either with the non-maneuvering vessel at anchor at “S” anchorage. After separation the Anchored /maneuvering vessel will return to a suitable position to transfer STS equipment to the workboat as required .

This area is normally considered to be operating at Security Level 1, so Oceane Fenders is unaware of any security concerns. However, it is a good practice not to allow unauthorized craft alongside at any time during

- the STS Operation, and in any case your vessel will need to remain in compliance with your own ISPS requirements.

Maximum weather operating limits for STS operations in the area:

| Parameters | Operations | Status / Actions |
| --- | --- | --- |
| Wind Speed 20 Knots or less Wave Height less than 2m Wave period Max 08Sec | Maneuvering | Safe |
| Wind Speed 20 knots or more Wave Height of 2.5 meters OR Wave Period Max 08sec | Cargo Transfer | Stop |
| Wind Speed 30knots or more Wave Height of 2.5 meters OR Wave Period Max 08sec | Preparation for cast off | Drain/disconnect and ready for Cast off |

Vessels will be provided with a weather forecast every 12 hours by STS Organizer.

Following equipment will be used for the forthcoming STS.

Enter No. of Fenders. x Primary Fenders

Enter No. of Fenders.  x Secondary Fenders

Enter No. of Hoses. x 10”x12

1 x Personnel Transfer Basket Select the applicability.

The mooring / unmooring sequence will be discussed and agreed between both the vessels.

The Latest Edition of OCIMF STS Checklist from 1 to 5 will be complied with / completed during the operation.

Emergency Communications & Spill Containment - Port Authority and MRCC

- Both vessels must be ready to depart at short notice, with Masters equally responsible for emergency planning. Shipboard emergencies are handled by the Masters as per the vessel’s SMS (including SOPEP/SMPEP), with Oceane providing assistance when requested:

- The POAC should alert Oceane operations Team.

- Oceane operations Team activates the Emergency Response Team (ERT) to coordinate with.

- If the Oceane POAC/Superintendent is incapacitated, immediately contact:

- Port Emergency Response Contact: Phone: +971 4527 4630; : Email:shipping@moei.gov.ae

PFSO: +971(09)2070279; Mobile No. +971(056)5114220 Email: sec_pof@fujairahport.ae

Marpol National Contacts: Tel: +971 24182240, +971 124182165; Email - Shipping@fta.gov.ae & Abdulla.Alhayyas@fta.gov.ae

Emergency Fire: Dial 997

Part B – Generic Information

Operational Risk Assessment

In case of any emergency or loss of containment, POAC will provide back up support, but your vessel’s contingency plans will take precedence. OGC will also activate its own Emergency Plan and assess its own response as appropriate.

- However, the following actions should be considered / taken, in the event of an emergency or loss of containment during an STS transfer operation

Raise the alarm and stop the cargo transfer.

Initiate emergency procedures and proceed to emergency stations. Always refer MSDS of the cargo and the contents of H2S prior to sending your crew to attend the loss of containment.

Drain and disconnect the cargo hoses most probably hoses will be transferred to the vessel with the lower freeboard. The following mitigation measures should be adopted throughout the STS operation.

Emergency signals agreed upon known and understood by both Vessels.

Fire - fighting equipment ready for immediate use.

Main engines ready for immediate use are always on both Vessels.

Cargo pumps, trips, and all other equipment relevant to the cargo transfer tested prior to the operation

Bridge watch to be maintained at all times.

Crew available and systems prepared to drain and disconnect hoses at short notice.

Loss of containment equipment prepared and ready for use.

Extra mooring tails / lines ready at mooring stations as replacements in case of breakage.

Fire axes are in place at mooring stations for cutting of lines, if necessary.

Fire axe and tools at the manifold; the fire axe to cut hose strap if necessary.

Reverse Lightering

- Reverse lightering is an STS (Ship-to-Ship) transfer operation where a smaller, loaded vessel moves alongside a larger, partly loaded or empty vessel to discharge cargo. Reference should be made to OCIMF Considerations Applicable to Reverse Lightering (Sept 2009).

- To carry out this operation safely :

- The approaching vessel must maintain a controlled speed, ideally less than 0.3 knots (During berthing with Tugs), and should approach at a 30–45° angle to minimize momentum and enable smoother alignment.

- Operating weather limits should be conservatively adjusted. Both vessels must stay informed of updated weather conditions throughout the operation.

- Mooring lines must be of adequate strength, correctly positioned, and routinely checked for tension and wear throughout the operation.

- Inspect chocks, fairleads, and all mooring equipment before commencement.

- Adjust mooring lines at least once per hour to accommodate cargo-induced draft changes.

- Consider limiting to daytime berthing only.

- Monitor rolling periods dynamically and adjust vessel heading to align with swell direction.

Berthing vessels of similar LOA

- During Ship-to-Ship (STS) operations involving vessels of similar lengths—defined as having less than a 10% difference in Length Overall (LOA)—additional precautions are essential due to limited margin for offset and increased contact risk.

- Operations should preferably be conducted during daylight hours to enhance safety and visibility. Unmooring, however, may be carried out at any time as deemed safe.

- Operating weather limits should be conservatively adjusted. Both vessels must stay informed of updated weather conditions throughout the operation.

- There should be a shorter span available for mooring lines, extra fore and aft lines may be necessary to ensure secure restraint. Vessels should be deliberately offset during mooring to prevent alignment of accommodation blocks or bridge wings, allowing the fenders to fully engage the parallel midship sections.

- Ensure minimal relative heading differences between vessels to prevent misalignment during contact. Monitor transverse thrust effects on maneuverability, especially with similar-sized vessels.

- Any reduction in the standard mooring pattern is strongly discouraged due to the tighter tolerances involved in operations between similarly sized vessels.

- Use a combination of headlines, stern lines, and springs to accommodate changes in freeboard and stability during cargo transfer. Monitor lines continuously for tension fluctuations caused by tidal shifts or cargo displacement

- Location Risk Assessment

Location Risk assessment has been provided for the location of STS Operations at Fujairah “S” Anchorage.

Emergency Communications – Designated Person Ashore Oceane Fenders DMCC

In the event of an emergency, the vessels’ contingency plans (including SOPEP/SMPEP) shall be followed.  In addition, the following personnel must be contacted without delay.

24 Hrs. Emergency Line: +971 4834 7310

Capt. Jagdeep Singh Sodhi

CEO / Designated Crisis Manager

+971 50 497 4021

Primary Contact

Secondary Contact

Mr. Sunil Kurup

Director / Alternate Designated Crisis Manager

+971 58 548 1979

Tertiary Contact

Reya Semira

Co-Ordinator

+971 54 511 1179

Cleofe

Alternate Co-Ordinator

+971 50 164 4311

Hajwane Walim

Assistant Crisis Manager /

Co-Ordinator

+971 56 958 5005

Mooring Master Roles and Responsibility

The role of the Mooring Master is not to relieve the masters of the vessels involved in the operation, of their duties or responsibilities. The Mooring Master is to ensure, through the provision of professional advice and guidance to the master’s the coordination and safe completion of the STS operation:

Review all pre-operation documentation once it is completed, and agreed with all parties, including but not limited to risk assessments, compatibility studies, fendering and mooring plans.

Inspect the STS equipment and fenders prior to use and ensure that it is fit for service.

Conducts a pre-operations discussion with the responsible persons of involved vessels, including lightering support vessels and tugs, as appropriate. Relay the Mooring Masters Standing Orders and post on the Vessel bridge for the Bridge Team to comply with.

Confirms that personnel involved in each part of the operation are properly briefed and understand their responsibilities. Conduct informal safety meetings (toolbox talks) with the officers and crew prior to commencing rigging the equipment.

Coordinate the loading and unloading of STS equipment.

Manage, prepare, and coordinate the rigging offenders.

Coordinate the mooring operations.

Confirm the safe connection of transfer hoses. Confirm that cargo transfer rates are being monitored together with associated vapor management system.

Keep Operations continuously informed about progress and milestones and advise on estimated schedule of operation going forward. Immediately report to Operations any anticipated or unforeseen events which may impact on the schedule of the operation.

Confirms and coordinates the safe disconnection of the transfer hoses.

Coordinate unmooring operations. Assist the Master and Pilot of the daughter Vessel (if applicable) during departure, until the vessels are clear of one another.

Conduct informal safety meetings (toolbox talks) with crew in preparation of unrigging fenders. Coordinate the unrigging of the fenders from the vessel. De-mobilize the fenders from the vessel, carry out post operation checks and provide reports to Operations.

Vessel Responsibility

- As per the STS Guidelines, conduct an emergency drill, where practicable, within 24 hours of the operation and in any case no more than 7 days before commencing the STS transfer operation.

Ensure that all required information is supplied promptly and accurately to ensure that the JPO is reflective of the true conditions of the vessel.

Provide the Mooring Master with the Pilot Card detailing the vessels maneuvering characteristics.

Passage planning and navigation.

Ensure that mooring equipment and lines are fit for use prior to operation.

Pre-operational cargo management requirements.

Bridge and Deck watchkeeping to be maintained at all times.

Machinery operation.

Monitor the weather forecasts and report to the Mooring Master during the operation and in accordance with the Mooring Master Standing Orders.

Monitor the fenders and moorings and report any changes to the Mooring Master.

Mooring and unmooring including line management during operations.

Fender management under the advice of the Mooring Master.

Officer to be in attendance and supervise vessel crew during fendering and un-fendering.

Preparation of the lift plan for landing equipment.

Operation of cranes.

Handling and rigging/unrigging during loading and unloading of equipment.

Officer to be in attendance and supervise vessel crew during hose connection and disconnection.

Management of transfer equipment under the advice of the Mooring Master.

Connection and disconnection of hoses under the advice of the Mooring Master.

Safety management includes broadcasting of safety messages as required.

Compliance with security requirements as per local area and vessel level.

Emergency operations and contingency planning.

Ensure that all required checks and rounds are carried out prior to and during cargo operations.

Area Specific requirements (if any)

Fendering

The Mooring Master will direct and supervise the fendering operations from onboard.  Please advise your officers and crew that the instructions given by the mooring master should be followed and be consistent with the safe operation of your vessel. Failure to follow the safe instructions given by the mooring master will result in the STS operation being aborted. Safe operations are paramount, and any concerns should be immediately addressed to the mooring master.

Navigational Watchkeeping During STS

General Navigation Safety dictates that when a vessel is underway or at anchor, a proper watch will be stood from the bridge. This is to be no different when conducting STS operations underway or at anchor.

The OCIMF Ship to Ship Transfer Guide addresses the issue in 6.2.3 “Once at anchor, each ship is responsible for keeping its own anchor watch (see Section 3.6.)”. It is expected that proper navigational watches will be kept on your vessels during the entire operation. The Mooring Master is not a part of the vessel’s complement and legally cannot be utilized to meet the definition of proper watch keeping, nor will Oceane Fenders allow them to engage in such activities.

Supervision of Critical Stages

Please ensure there is an OOW in attendance at the following critical stages, where applicable:

- Fendering and un-fendering.

- Mooring Forward, Aft, and Bridge.

Hose connection and hose disconnection.

Hose draining.

Unmooring Forward, Aft and Bridge

Pre-Transfer Meeting

After the vessels have completed mooring operation the preparation for connection of transfer equipment will take place. Upon completion of the transfer equipment, pre-discharge meeting will be held via VHF and will include but not limited final acceptance of the JPO, completion of the safety checks list etc.

During the Pre-Discharge meeting the below shall be filled out and confirmed by all parties involved in the operation. This outlines when the STS Mooring Master should be informed, and when actions are required. The JPO outlines the continuous checks and rounds which are required by the vessel during an STS Operation regardless of whether the Mooring master is present or not. Failure to do so may result in severe incidents.

Weather Guidelines

Special attention shall be given to electrical storms and squalls. When lightning and squalls are expected to be, or are within the vicinity of the STS transfer, they are to have particular attention paid to the relative direction, track, and speed. If the lightning/squalls are due to be passing over the STS location, or if any doubt exists with regards to the track of the lightning/squalls, then the flow of cargo between the ships shall be suspended until clear.

Service Craft and Launches

No unauthorized craft will be allowed alongside either vessel throughout the transfer.

The loading of stores, bunkering and or provisions are prohibited during the transfer operation (Unless agreed with the STS operations department). If you have a requirement, please inform the mooring master and we will advise accordingly.  It is prohibited to transfer stores or provisions from one vessel to the other.

Security Requirements

All security arrangements will be as per that required under the ISPS code and in line with the Security Policies of each vessel.

Checks During Operations

Manifold and Cargo Transfer

Ensure a crew member is always at the manifold.

Maintain an exchange of cargo transfer rate and quantity transferred with the other vessel(recommended hourly but this can be discussed between both vessels at the pretransfer meeting).

Call the Mooring Master if there are any changes or deviations from the cargo transfer plan.

Call the Mooring Master if there are any changes or deviations from the cargo transfer plan.

Moorings, Fenders, and Portable Chocks

Moorings and fenders should be inspected frequently and tended accordingly. This includes the greasing of chocks and moorings and/or the condition of any anti chafe protection in place. When using portable chocks, the secondary fender moorings must be inspected more frequently. If any of the following is observed inform the Mooring Master.

- Any moorings fail (Cargo must be stopped if in progress).

The other vessel does not appear to be tending its moorings.

Vessels experience increased movement.

Any damage to the fenders, fender moorings or associated equipment.

Excessive movement of fenders.

Excessive movement of portable chocks or they appear to be working loose.

Any doubt to the position or condition of any moorings, fenders, or chocks.

Hoses

Hoses and their lashings should be inspected frequently during cargo operations. 
Inform the Mooring Master if:

- Hose connections are leaking.

There is excessive movement of the hoses.

If there is any doubt about the positioning of the hoses.

Damage of any kind is noticed.

Transfer Area

During the operation, it is important that a keen eye of the transfer area is maintained. Inform the Mooring Master if:

- Traffic in the area causes a concern.

The vessels appear to be leaving the designated transfer area.

Local authorities are asking for information the bridge team cannot provide.

Navigational hazards present themselves.

Any other times the bridge team feel uncomfortable with a situation

Emergencies

Mooring Masters responsibility during emergencies

The Masters of the vessels involved in the operations are ultimately responsible for all actions carried out by their vessel and crew during emergency situations. The Mooring Master can, as required, coordinate support craft and give advice as to the best course of action regarding operations and vessel handling during an emergency.

Notice for engines

Both vessels will maintain their engines and steering gear in a constant state for readiness in case of emergency.

Emergency signals

Emergency Signals between the vessels will be agreed during the pre-loading safety meeting and confirmed in the ship / shore checklist.

Stop Work (Calling an All Stop)

Any unsafe action, position, equipment, person and procedure gives all the right to the mooring master to implement STOP WORK and Oceane Fenders cannot be held liable for any delay occurred due to halted operation.

“STS SPECIFIC STOP WORK EVENTS

Prior to an STS operation there is specific criteria that the company considers to be cause for a stop work intervention:

- Vessels not in compliance with the agreed JPO.

Vessels not in compliance with STS checklists.

Operation not in compliance with JPO criteria.

Vessel essential equipment failure even if rectified immediately including main engine failure or steering loss or inability of the vessel to maneuver as required.

Noted defect from vessel that may compromise the safety of the operation including crane failure.

Poor performance of the vessels crew including helmsman or Captains.

Crew failing to keep adequate deck, engine, or bridge watch during operations.

Crew failing to complete safety rounds as required.

In the event of any emergency onboard either vessel.

In the event of a security concern.

If the pilot is putting the vessel into a situation where an incident may occur.

If members of the bridge team, including the pilot engage in communication that do not allow the other members of the bridge team to fully understand the operation and instructions being given.

How to Stop Work

The following outlines the steps required to stop work under the Stop Work Authority Requirements:

- When a perceived unsafe condition is identified as per the above Stop Work Events, they must immediately initiate a stop work intervention with the person(s) potentially at risk.

If the affected person(s) is not at immediate risk and the supervisor is readily available, the stop work action should be coordinated through the supervisor. If the supervisor is not readily available or the affected person(s) are at immediate risk, the stop work intervention should be initiated directly with those at risk.

Stop work interventions should be initiated in a positive manner by briefly introducing yourself and starting a conversation with, “I am using my stop work authority” because using this phrase will clarify the user’s intent and set proper expectations.

Notify affected personnel and supervision of the stop work issue. If necessary, stop associated work activities, remove person(s) from the area, stabilize the situation, and ensure the area is as safe as possible.

Affected parties shall discuss and gain agreement on the stop work issue. · Report the stop work to Oceane Fenders DMCC.

If determined and agreed that the stop work issue is valid, then every attempt should be made to resolve the issue prior to starting work.

If a Mooring Master has aborted an operation, he should contact the operator and wait for advice on when to resume.

Resumption of work

In the event of a stop work being actioned, the work / operation can only continue once the following has been satisfied:

- The unsafe condition / act has been risk assessed and appropriate controls put in place, as required to, as far as reasonably practicable, prevent any further occurrences.

Any work permits related to the work which has been stopped and any other affected jobs / operations are required to be reissued.

COO / CEO gives authorization to carry on work.

Suspension of cargo operations

Examples of emergency situations which require suspension of cargo operations, and the calling of the Mooring master are:

- Accidental cargo release and/or gas accumulation on deck.

Parting of a mooring line.

Onset of adverse weather or environmental conditions.

Ships emergency.

Cargo hose leak/failure.

Insurgent activity.

Loss of communication between vessels.

Breakaway criteria

Criteria for an emergency breakaway can consist of the following:

- Unexpected deterioration in wind, wave, or swell conditions.

Primary fender failure.

Mooring System failure.

Cargo hose failure.

Cargo Containment failure.

Vapor release.

Any other shipboard emergency which presents a risk to either ship involved in the operation.

Local area emergency requiring emergency departure of vessels.

Emergency Hose Disconnection

The other vessel must be notified when a vessel deems it necessary to breakaway to ensure that the manifold area is clear of personnel. If the situation allows, the manifold should be correctly and timely disconnected. Mooring squads and manifold disconnection personnel must be mustered and arrangements for safe and controlled breakaway made.

The contingency plans in the risk assessment and emergency response plan must be followed where possible to ensure a safe breakaway is undertaken. Previously highlighted contingency anchorages should be utilized.

Communications failure

During cargo operations, essential personnel on both ships should always have a reliable, common means of communication, including a spare system.

During cargo operations, in the event of breakdown of communications on either ship the emergency signal should be sounded and all operations in progress should be suspended immediately. Operations should not be resumed until satisfactory communications have been re-established. The Mooring Master is to be informed immediately.

If the primary means of communication fails, the vessels shall attempt to communicate using the secondary means of communication. Should this too be inoperable then the transfer operation shall immediately be suspended. The operation should only be resumed once both primary and secondary means of communication have been restored, or alternative primary and secondary means of communication are identified and agreed on.

If the communication failure occurs during the maneuvering stage of the operation, the maneuvering should be aborted with the intentions being communicated by use of the appropriate sound signal.

Contingency Plans and Emergency Spill and Containment Procedures

The Contingency Plans for the operation will be as per the vessel procedures. This will include an emergency contact list of which shall be posted on board all vessels involved in the operation. The Contingency Plans should have relevance to the location of the operation and consider the resources available.

The Emergency Spill and Containment Procedures should ensure that reference be made to the information provided in the Material Safety Data Sheet (MSDS) of the products being handled.

Consideration should be given to:

- Procedures for raising the alarm.

Cessation of operations during emergencies.

Notification procedures.

Emergency stations and preparations to initiate emergency procedures.

Deployment of mooring gangs to stations.

Emergency disconnection of cargo transfer equipment.

Preparation of engines for maneuvering.

Unmooring

Each vessel contingency plans are to be agreed prior to operation commencing and both shall agree to an emergency being indicated by sounding the ships internal alarm signal and by sounding five or more short blasts on the ships whistle to warn the other ship.

Both vessels involved in the operation should be always at an advanced state of readiness, this is including, but not limited to:

- Main engines are in either standby mode or on short notice of readiness.

Cargo pump and all other equipment trips relevant to the transfer tested prior to the operation.

Minimum of three (3) Crew available and systems prepared to drain and disconnect hoses at short notice.

Spill management equipment prepared and ready for use.

Mooring equipment ready for immediate use and extra mooring lines ready at mooring stations as replacements in case of breakages.

Fire-fighting equipment ready for immediate use.

Tug/Support craft readiness (if utilized).

|  | Joint Plan of Operation | Form No: | OPS-OFD-006 |
| --- | --- | --- | --- |
|  |  | Rev.No. | 1.0 |
|  |  | Issue Date: | 01 Mar 2023 |
|  |  | Approved by: | SK |
|  |  | Page: | 1 of 2 |

|  | Joint Plan of Operation | Form No: | OPS-OFD-006 |
| --- | --- | --- | --- |
|  |  | Rev.No. | 1.0 |
|  |  | Issue Date: | 01 Mar 2023 |
|  |  | Approved by: | SK |
|  |  | Page: | 1 of 2 |